# Components List

ESP32 Smart Environmental Monitoring System

This file documents the hardware and software used in the working ESP32 environmental-monitoring prototype. The system reads temperature and humidity, reads the MQ-135 analog output, displays readings on an OLED, and connects to Wi-Fi and Blynk. Firmware also includes a local web dashboard.

# 1. Hardware Components

| # | Component | Qty. | Purpose / notes |
| --- | --- | --- | --- |
| 1 | ESP32 development board | 1 | Main microcontroller; Wi-Fi connectivity and sensor processing. |
| 2 | DHT11 temperature and humidity sensor/module | 1 | Measures temperature and relative humidity; DATA connects to GPIO 4 in the current sketch. |
| 3 | MQ-135 gas/air-quality sensor module | 1 | Analog output goes to GPIO 34. Raw ADC value only; not a calibrated AQI or ppm reading. |
| 4 | I²C OLED display (128×64; SSD1306-compatible if that is the installed model) | 1 | Displays local readings; current firmware uses SDA GPIO 21, SCL GPIO 22, address 0x3C. |
| 5 | Buzzer / compatible buzzer module | 1 | Audible alert; control signal assigned to GPIO 25. Check voltage/current rating. |
| 6 | LED | 1 | Status / GPIO output and PWM testing; GPIO 18 in the sketch. |
| 7 | Current-limiting resistor for LED | 1 | Limits current; keep in series with a bare LED. |
| 8 | Breadboard | 1 | Prototyping and temporary connections. |
| 9 | Jumper wires | As needed | Connects ESP32 and modules. |
| 10 | USB data cable compatible with ESP32 board | 1 | Firmware upload and development power. |
| 11 | Wi-Fi network or 2.4 GHz hotspot | 1 | Network connection for Blynk and the local dashboard. |

# 2. Pin Reference From the Current Sketch

| Module / signal | ESP32 pin |
| --- | --- |
| DHT11 DATA | GPIO 4 |
| MQ-135 analog output | GPIO 34 (ADC input) |
| Buzzer control | GPIO 25 |
| LED control / PWM | GPIO 18 |
| OLED SDA | GPIO 21 |
| OLED SCL / SCK (I²C clock on this module) | GPIO 22 |

Important: Verify the labels and pin order printed on each physical module before changing wiring. OLED labels GND, VDD, SCK, SDA match the table only when the module is configured for I²C; module versions can differ.

# 3. Software and Online Services

| Tool / library | Purpose |
| --- | --- |
| Arduino IDE | Edit, compile, and upload ESP32 firmware. |
| ESP32 Arduino board package | Enables ESP32 boards in Arduino IDE. |
| WiFi.h | Connects ESP32 to Wi-Fi. |
| BlynkSimpleEsp32.h / Blynk IoT Console | Sends and displays monitoring data remotely. |
| WebServer.h | Hosts the local web dashboard on the ESP32. |
| Adafruit DHT sensor library | Reads DHT11 temperature and humidity. |
| Adafruit Unified Sensor | Library dependency for applicable DHT library versions. |
| Adafruit GFX Library | Graphics functions for OLED. |
| Adafruit SSD1306 Library | Drives compatible SSD1306 OLED displays. |

# 4. Safety and Calibration Notes

Disconnect USB power before changing connections.

Do not connect ESP32 GPIOs directly to signals above 3.3 V.

MQ-135 modules are often powered from 5 V and their analog output may exceed the ESP32 safe input range. Check your exact module and use suitable voltage conditioning/divider if needed before connecting to GPIO 34.

Check the buzzer rating. A buzzer that draws too much current for a GPIO needs a suitable driver circuit.

MQ-135 readings need warm-up and calibration. The raw value in this project must not be described as certified AQI or ppm.

Never publish Wi-Fi passwords, Blynk Auth Tokens, API keys, or other credentials in a public repository. Use placeholders in public source code and rotate any credentials already exposed publicly.

# 5. Project Status

The project owner reports that the ESP32, DHT11, OLED display, Wi-Fi, and Blynk Console integration are working. This list reflects the components and pins visible in the current firmware; update it if the physical build changes.